# ESP32 Firmware Audit & PZEM Troubleshooting

# 1. Serial Log Analysis

The logs show a Task Watchdog (task_wdt) trigger on CPU 1. This occurs when the loopTask takes more than the allotted time (default 15s-30s) to reset the watchdog. The primary cause is the blocking HTTPS requests to Supabase happening sequentially in runCloudSync().

The logs also show consistent PZEM timeouts (addr=1 to 4), indicating the firmware is unable to communicate with the energy meters.

# 2. Software Audit & Fixes

• Watchdog Feeding: Added explicit esp_task_wdt_reset() calls between each network request in runCloudSync() to prevent reboots during slow internet connections.

• PZEM Pin Configuration: Detected a potential TX/RX swap in config.h. The standard for UART2 is TX=17, RX=16, but the project uses 14 and 27. Cross-wiring must be verified.

# 3. Hardware Troubleshooting Steps

• AC Power: Ensure all PZEM-004T meters are connected to AC mains. They will not respond via 5V USB alone.

• Wiring Cross: Verify TX (Pico) -> RX (PZEM) and RX (Pico) -> TX (PZEM).

• Grounding: Ensure a common GND exists between the ESP32 and all PZEM modules.